Icon Updating

- Hold pressing PTT key and PF2 key (the bottom side key), at the same time power on radio, LCD display UPDATE MODE

- Use PC cable connect PC to radio

- Double click on QXCodePro_Update.exe to run it

- Click Open Update File，choose D868_new_usb.spi

- Com Speed  921600, tick on Duplex

- Click Write to update the radio the latest firmware.

- Reboot the radio when the update is finished.