- 1. turn off radio, insert program cable.

- 2. Press and hold the side key 1 and the side key 2 to turn on radio, the indicator light is on, indicating radio into upgrade mode.

- 3. Start the upgrade software on the computer "Update.exe"

- 4. Click "Brows" to select the file to be upgraded, for example: GD-77_V2.6.7.sgl,

- 5. Click "Download" until the download is successful

- 6. Unplug program cable, turn off radio or reboot